# SERVO ULTRA 10W

DESCRIPTION

Servo Ultra 10W is a heavy duty diesel engine oils, and are blended from highly refined, high viscosity index base stocks and contain appropriate dosages of detergent-dispersant and antioxidant additives to meet the stated performance standards.

# APPLICATION

Servo Ultra 10W is recommended for all classes of supercharged diesel engines of high speed, high output such as those in:

- Heavy duty diesel vehicles / Tractors

- Earth moving / Off-highway equipment

- Transmission systems of Caterpillar equipment

- Large generating sets

Servo Ultra 10W meets the requirements of hydraulic fluid Type C2 specification of Detroit Diesel Allison, Division of General Motors Corporation, USA.

# PERFORMANCE BENEFITS

Servo Ultra 10W

- Effectively combat the deleterious products of combustion associated with high sulphur fuels, thus minimising deposits and wear.

- Maintain excellent engine cleanliness.

- Minimise deposits resulting from high output, low speed, low temperature operations.

- Provide excellent control of cold sludge and corrosive wear.

- Possess excellent oxidation stability ensuring long service life especially in transmission systems.

# PERFORMANCE STANDARDS

Servo Ultra 10W meet the following specifications:

- API CD

- US MIL-L-45199B

- Caterpillar Series 3

- E-DL 2 of IS:13656-2002 specifications

Servo Ultra 10W is approved by the leading manufactures including

- Bharat Earth Movers Ltd., Bangalore

- Escorts Ltd., Faridabad

# CHARACTERISTICS

| SAE GRADE | 10W |
| --- | --- |
| Kinematic Viscosity cSt @ 100C | 5Min. |
| Viscosity Index, Min. | 100 |
| Flash point (COC), C Min. | 200 |
| Pour Point, C Max. | (-) 27 |
| TBN mg KOH/gm | 9.5 – 11.5 |

Jul-26