SERVOFILM 32

DESCRIPTION

Servofilm 32 is a highly refined straight mineral oil having excellent water separation characteristics.

These are formulated out of solvent refined paraffinic type high viscosity index base stocks having inherent oxidation and thermal stability.

# APPLICATION

Servofilm 32 is suitable for use as Rubber Process Oil, bearing oils in steel mills.  These oils are also recommended for use in circulation systems where non-additive type lubricants with high demulsibility characteristics are required.

# PERFORMANCE STANDARDS

Servofilm 32 meets the following specification:

- IPSS 1-09-001-81

- Morgan Construction Co. USA

# PERFORMANCE BENEFITS

Servofilm oils

- Have good resistance to formation of deposits and sludge at normal operating temperatures

- Rapidly separate from water

- Recommended for use in Rubber processing where paraffinic type oil is required.

# CHARACTERISTICS

| ISO VG | 32 |
| --- | --- |
| ASTM, Colour, D – 1500 | 2 Max |
| Kin. Vis. cSt @ 40C, D – 445 | 29– 33 |
| Viscosity Index, Min, D – 2270 | 95 |
| Flash Point (COC), C Min, D – 92 | 190 |
| Pour Point, C Max, D – 97 | 0 |
| Aniline Point, D - 611 | 97 (Typ) |

Jul-26